Sizing is the general term for separation of particles according to their size.

The simplest sizing process is screening, or passing the particles to be sized through a screen or number of screens. Screening equipment can include grizzlies,[3] bar screens,wedge wire screens, radial sieves, banna screens, multi-deck screens, vibratory screen, fine screens, flip flop screens and wire mesh screens. Screens can be static (typically the case for very coarse material), or they can incorporate mechanisms to shake or vibrate the screen. Some considerations in this process includes the screen material, the aperture size, shape and orientation, the amount of near sized particles, the addition of water, the amplitude and frequency of the vibrations, the angle of inclination, the presence of harmful materials, like steel and wood, and the size distribution of the particles.

Classification refers to sizing operations that exploit the differences in settling velocities exhibited by particles of different size. Classification equipment may include ore sorters, gas cyclones, hydrocyclones, rotating trommels, rake classifiers or fluidized classifiers.

An important factor in both comminution and sizing operations is the determination of the particle size distribution of the materials being processed, commonly referred to as particle size analysis. Many techniques for analyzing particle size are used, and the techniques include both off-line analyses which require that a sample of the material be taken for analysis and on-line techniques that allow for analysis of the material as it flows through the process.

Historically the earliest method used,particles can be classified based on their specific gravity. Air is the main fluid medium used for the process. Gravity concentration processes include:

- Heavy media or dense media separation (these include, baths, drums, larcodems, dyana whirlpool separators, and dense medium cyclones)

- Shaking tables, such as the Wilfley table[4]

- Spiral separators

- Reflux Classifier

- Batac jigs

- Centrifugal bowl concentrators, such as the Knelson concentrator

- Jig concentrators are continuous processing gravity concentration devices using a pulsating fluidized bed.

- Multi gravity separators (Falcon, Knelson, Mozley and the Kelsey Jig)

- Inline pressure Jigs

- Reichert Cones

These processes can be classified as either dense medium separation or gravity separation. The difference between the two that gravity separation does not use a dense medium to operate, only water or air. Dense medium separation can be performed with a variety of mediums. These include, organic liquids, aqueous solutions, suspensions in water and suspensions in air. Of these, most industrial processes use suspensions in water. The organic liquids are not used due to their toxicity and difficulties in handling. The aqueous solution as a dense medium is used in coal processing in the form of a belknap wash and the suspension in air is used in water-deficient areas, like china, where sand is used to separate coal from the gangue minerals. The dense medium separation is also classified as absolute gravity separation as the sinks and the floats travel in different directions. The gravity separation is also called relative gravity separation as they separate particles due to their differences in the magnitude of the particle response to a driving force.

These processes can also be classified into multi-G and single G processes. The difference is the magnitude of the driving force for the separation. Multi-G processes allow the separation of fine particles to occur and these particles can be in the range of 10 to 50 micron. The single G process are only capable of processing particles that are greater than 80 micron in diameter.

Of the gravity separation processes, the spiral concentrators are one of the most economical due to their simplicity and use of space. They operate by flowing film separation and can either use washwater or be washwater-less. The washwater spirals separate particles more easily but can have issues with entrainment of gangue with the concentrate produced..

Froth flotation is an important concentration process. This process can be used to separate any two different particles and operated by the surface chemistry of the particles. In flotation, bubbles are introduced into a pulp and the bubbles rise through the pulp. In the process, hydrophobic particles become bound to the surface of the bubbles. The driving force for this attachment is the change in the surface free energy when the attachment occurs. These bubbles rise through the slurry and are collected from the surface. To enable these particles to attach, careful consideration of the chemistry of the pulp needs to be made. These considerations include the pH, Eh and the presence of flotation reagents. The pH is important as it changes the charge of the particles surface and the Eh affects the chemisorption of collectors on the surface of the particles.

This process was invented in the 19th century in Australia. It was used to recover a sphalerite concentrate from tailings, produced using gravity concentration. Further improvements have come from Australia in the form of the Jameson cell, developed at the University of Newcastle, Australia. This operated by the use of a plunging jet that generates fine bubbles. These fine bubbles have a higher kinetic energy and as such they can be used for the flotation of fine grained minerals, such as those produced by a mill.

تا هر جا كه رسيديد ترجمه كنيد!!!