•	The secondary and backscattered electrons are used for imaging while the X-rays give characteristic chemical information of the emitting atoms.

Factors that affect SE emission

3.	Atomic number (Z)

•	More SE2 are created with increasing Z

•	The Z-dependence is more pronounced at lower beam energies

4.	The local curvature of the surface

(the most important factor)

Backscattered electrons (BSE)

•	A fraction of the incident electrons is retarded by the electro-magnetic field of the nucleus and if the scattering angle is greater than 180 ° the electron can escape from the surface

•	Back scattered electrons(BSE) consist of high-energy electrons originating in the electron beam, that are reflected or back-scattered out of the specimen interaction volume by elastic scattering interactions with specimen atoms.

•	Since heavy elements (high atomic number) backscatter electrons more strongly than light elements (low atomic number), and thus appear brighter in the image.

•	High energy electrons (elastic scattering)

•	Fewer BSE than SE